CALCULATIONS :-

Conversion Gain of Gilbert Cell Mixer(RF → IF) :-

Conversion Gain (CG) = IF output voltage (or power) / RF input voltage (or power)

- Identify RF input amplitude (known)

RF input peak amplitude = 5 mV

RF input RMS:                VRF,rms = 5 mv / 2  = 3.54 mV

- Identify IF output amplitude (from waveform)

Differential IF waveform peak ≈ ±2.2 mV

So, peak-to-peak ≈ 4.4 mV

Convert to RMS -            V1F,rms = Vpp / 22  = 4.4 mV / 22   ≈ 1.56 mV

- Conversion Gain (Voltage)

CGv = VIF,rm/VRF,rms= 1.56mV/3.54mV = 0.44

Convert to dB-                 CGdB​=20log10​(0.44)=−7.1 dB

IF Voltage Gain Calculation:

AV  = 20 log10 ( VIF,out / VIF,in)

VIF,in  ≈ 1.6 mV (RMS)

VIF,out ≈16mV (RMS)

AV  = 20 log10 ( 10) = 20 dB