Results :-

- Mixer-Only Results (Before IF Amplifier)

| Parameter | Value | Notes |
| --- | --- | --- |
| RF Input Frequency | 2.4 GHz | Applied at mixer RF port |
| LO Frequency | 2.395 GHz | Differential LO drive |
| IF Frequency | 5 MHz | ¦RF − LO¦ |
| RF Input (RMS) | 3.54 mV | Measured at mixer input |
| IF Output (RMS) | 1.56 mV | Differential output (Vdiff) |
| Conversion Gain | −7 dB | 20·log(IF/RF) |
| Supply Voltage(VDD) | 1.8 V | PTM 180 nm |
| Topology | Double-Balanced Gilbert Cell | Differential |
| Verification Method | Transient + FFT | LTspice |

Table 1: Gilbert Cell Mixer Performance Summary (Pre-IF Amplification)

- Frequency-Domain Verification (Mixer Stage)

| Metric | Observation | Metric |
| --- | --- | --- |
| Dominant Spectral Component | 5 MHz IF tone | Dominant Spectral Component |
| RF Leakage | Strongly suppressed | RF Leakage |
| LO Feedthrough | Strongly suppressed | LO Feedthrough |
| Noise Floor | ≈ 80–100 dB below IF | Noise Floor |
| FFT Signal Used | V(vdiff) | FFT Signal Used |
| FFT Window | Steady-state (µs range) | FFT Window |

Table 2: FFT-Based IF Verification at Mixer Output

- IF Amplifier Performance (Post-Mixer)

| Parameter | Value | Notes |
| --- | --- | --- |
| IF Input Source | V(vdiff) | Differential output of Gilbert mixer |
| IF Center Frequency | 5 MHz | ¦RF − LO¦ = ¦2.4 GHz − 2.395 GHz¦ |
| IF Input (RMS) | ~1.5–2 mV | From mixer differential output |
| IF Output (RMS) | ~15–20 mV | Measured at IF_OUT |
| IF Voltage Gain | ≈ 20 dB | 20·log(Vout/Vin) |
| Supply Voltage | 1.8 V | Same as mixer |
| Technology | PTM 180 nm CMOS |  |
| IF Amplifier Topology | Common-Source (CS) | Single-ended |
| Load Resistance (RD) | 1.5 kΩ | Drain load |
| Bias Voltage (Vbias_IF) | 0.6 V | Gate bias |
| Coupling Capacitors | 10 nF | Input & output AC coupling |
| IF Bandwidth | >10 MHz | Sufficient for 5 MHz IF |
| Verification Method | Transient + FFT | LTspice |

Table 3: IF Amplifier Performance Summary